Assessing spatial variability of nutrients, phytoplankton, and related water quality constituents in the California Sacramento-San Joaquin Delta during treated wastewater effluent holds: August and September 2019 high resolution mapping surveys

# Methods

The data set comprises four days of high-resolution mapping and discrete sample collection of nutrients, phytoplankton, and related water quality constituents conducted in the Sacramento River, Georgiana Slough, and the North and South Forks of the Mokelumne River on August 28th and September 10th through 12th, 2019, coincidental with planned holds of treated wastewater effluent from Sacramento Regional Wastewater Treatment Plant (SRWTP). SRWTP halted treated effluent releases to the Sacramento River at their outflow site (38° 27’ 15”N, 121° 30’ 00”W), located just south of Freeport Bridge, from 23:54 on August 26th through 01:25 on August 28th, 2019, and from 23:57 on September 9th through 01:19 on September 12th, 2019. The purpose of this study was to observe changes in nutrients, phytoplankton, and related water quality constituents in the different waterways surveyed under conditions representing (i) typical contribution from SRWTP effluent, (ii) the transition from typical SRWTP effluent conditions to SRTWP effluent-free conditions, and (iii) SRTWP effluent-free conditions.

## Definition of regions and reaches for analysis

To evaluate temporal and spatial trends and to identify controls on nutrients, phytoplankton, and other water quality parameters at the regional scale, we defined ‘Study Regions’ based on hydrodynamically distinct zones identified by Jabusch and others (2016). Sample collection for this data release took place in the North Delta and Mokelumne System tidal transition zones. A ‘transition zone’ is defined where transport is dominated by tidal forcing.

## Sampling and analysis

### Underway high resolution (HR) water sampling and onboard analyses

Surveys were conducted using the USGS R/V Mary Landsteiner on August 28th and September 10th through 12th, 2019, following the approach described by Fichot and others (2016), Downing and others (2016), Kimmerer and others (2019), and Stumpner and others (2020). On each day, data were collected during daylight hours. For analysis, sample water was continuously pumped onto the boat using two SHURflo Aqua King II pumps at a pressure of 55 psi and rate of 5 GPM (SHURflo Pentair, London, United Kingdom) while underway at speeds up to 30 mph (13 m/s) using a pick-up tube mounted at a fixed depth of approximately 1 m below the surface, routed through a 178 μm in-line strainer (Cole Parmer; EW-29595-47, Vernon Hills, IL) to remove large debris and into a pressure-compensated manifold that maintained system pressure at a proscribed level irrespective of boat speed (Downing and others, 2016). A 2-stage debubbler was used to remove bubbles that can interfere with optical measurements. Sample water was split into separate flow paths for each analytical flow path, with constant flow rates maintained by metering the discharge line, except for the flow path serving the open split interface, which was metered on the feed line. Flows were continuously monitored using a sight gauge (0.1-1 GPM Flow Meter LZM-15 with adjustable knob) on each flow path.

The manifold delivered water to the following flowpaths: (1) A flow-through system consisting of a thermosalinograph that recorded temperature and conductance (Sea-Bird Scientific SB45 (TSG), Belluve, WA); fluorometers that measured fluorescence of chlorophyll (fCHL) and fluorescence of dissolved organic matter (fDOM, WETLabs WETstar (WS), Philomath, OR); a beam transmissometer that recorded transmittance and attenuation (WETLabs model C-Star transmissometer (CStar), Philomath, OR), a nitrate (NO3) analyzer (SUNA V2; Sea-Bird Scientific,

Bellevue, WA). (2) A flow chamber for a multiparameter water quality sonde (YSI EXO2; Xylem Inc. (EXO), Rye Brook, New York) equipped with sensors to measure temperature, specific conductance (SpC), turbidity, pH, dissolved oxygen (DO), fDOM and fCHL that then passed water to a fluorometer designed to measure different algal classes (FluoroProbe III (FP); BBE Moldaenke, Kiel, Germany; Beutler and others, 2002). (3) An open-split interface at atmospheric pressure that served water filtered through a 0.2 micron high-capacity in-line filter (Suez Memtrex, 25 cm length, MNY921EGS) to the on-board ammonium (NH4) analyzer.

The NH4 analyzer was a continuous flow, gas diffusion/conductivity-based (Carlson, 1978) analyzer for ammonium (NH4) analysis (TL-2800; Timberline Instruments, Boulder, CO) that was modified for field operation and continuous data collection by the manufacturer (Thompson and others, 2023). Modifications included installation in a ruggedized housing, addition of an automated line-switching valve, addition of a heating unit to maintain the instrument at a constant above-ambient temperature, and changes to the software. The analyzer was run in continuous mode with frequent periodic introduction of deionized organic-free water (OFW, resistivity >18.2 MΩ/cm), and standard solutions to continuously assess instrument performance and to correct for baseline drift during the day. Full standard curves were run at the beginning and end of each day, and partial curves were run throughout the day each time the boat stopped to sample.

All instrumentation was cleaned, and calibrations were checked prior to each use following the manufacturer's recommendation or as described below. Data for most instruments were recorded at 1-second frequency on a single data logger (CR6, Campbell Scientific, Logan, Utah) together with a timestamp and boat position obtained from a high-resolution global positioning system receiver (16X-HVS, Garmin, Olathe, Kansas). The FluoroProbe logged data internally and to the host software every 4 seconds. The NH4 analyzer was connected to a stand-alone computer and collected data through its native software. All data were displayed in real time, so scientists on board could respond when they observed changes relevant to study objectives, or if issues with flow or instrument performance were noted. Data collected in situ are referred to as high resolution (HR) parameters.

### Discrete water sampling (DS) and laboratory analyses

Discrete water samples (DS) were collected either underway or–in the case of more extensive sample requirements–while stopped on station. Discrete samples were collected while underway either using a fourth flow path on the sampling manifold, or using a separate system with a separate pick-up tube mounted adjacent to the main sampling pick-up and pumped through a 0.2-micron filter (Suez Memtrex, 25 cm length, MNY921EGS) into a sample bottle. At designated sampling stations (approximately 10 per day) samples for determination of NH4, NO3, PO4, dissolved organic carbon (DOC), and total dissolved nitrogen (TDN) concentrations were collected using this system. Also at these stations unfiltered samples were collected approximately from 1 m depth using a submersible pump for analysis of chlorophyll a (Chl a), pheophytin, phytoplankton enumeration, and picocyanobacterial counts. Samples for phytoplankton enumeration and picocyanobacteria counts were fixed immediately with Lugol’s iodine solution (4 mL) and formaldehyde (1 mL), respectively.

All samples were stored on ice in the dark during transit to the laboratory where they were stored at 2°C. Dissolved organic carbon samples were preserved to <2 pH with high-purity sulphuric acid the day following collection and stored at 2°C. Samples for pigment analysis were filtered within 24 hours of collection and immediately frozen (-80°C). Total Chl a samples were filtered through a 0.3 μm nominal pore size glass fiber filter (Advantec MFS, Inc, Dublin, CA). Chl a associated with the larger sized cell fraction were filtered through a 5 μm nominal pore size 25 mm polycarbonate filter (General Electric Healthcare, Chicago, IL). Frozen Chl a samples and chilled nutrient samples were shipped to the USGS National Water Quality Laboratory (Denver, CO). Picocyanobacteria and phytoplankton enumeration samples were shipped to BSA were shipped to BSA Environmental Services, Inc. (Beachwood, OH) for analysis.

Concentrations of nutrients were determined colorimetrically using an automated-segmented flow analyzer. Concentrations of nitrogen as nitrite (NO2-N) and as nitrate plus nitrite (NO3-N + NO2-N) were determined by colorimetric analysis (Fishman, 1993; Patton and Kryskalla, 2011), ammonium as nitrogen (NH4-N) was determined by colorimetric analysis after reaction with salicylate-hypochlorite (Fishman, 1993), and total dissolved nitrogen (TDN) was determined by alkaline persulfate digestion (Patton and Kryskalla 2003). Orthophosphate as phosphorous (PO4-P; also referred to as soluble reactive phosphate) was determined by colorimetric analysis after reaction with NH4 molybdate and reduction with ascorbic acid (Patton and Truitt, 1992).

Chl a and phaeophytin concentrations were determined according to EPA Method 445.0 (Arar and Collins, 1997) using sonication extraction.

Dissolved organic carbon concentrations were measured using a total organic carbon analyzer

(TOC-VCSH, Shimadzu Scientific Instruments, Columbia, Maryland) using high-temperature catalytic combustion according to a modified version of USEPA method 415.3 (Potter and Wimsatt, 2012).  The accuracy and precision of these measurements were within data quality objectives as indicated by an internal laboratory standard (caffeine), laboratory replicates, and matrix spikes. The laboratory reporting limit for DOC concentration was 0.30 mg C/L based on three times the standard deviation of a low concentration standard measured over an annual cycle.  Absorbance spectra and fluorescence EEMs were simultaneously measured using an Aqualog® equipped with a photodiode and charge-coupled device (CCD) (Horiba Instruments, NJ, USA) according to the method described by Hansen and others (2018). Briefly, excitation and absorbance spectra were measured using a 150 W xenon lamp, a 5 nm bandpass, and a 1 s integration time at wavelengths of 240–600 nm. Emission spectra were collected with a CCD at approximately 1.64 nm (4 pixel) intervals at wavelengths of 250–600 nm. To reduce ultraviolet light exposure of the sample and limit the effects of photobleaching, excitation and absorbance wavelengths were scanned from low to high energy (i.e., visible to ultraviolet range) during analysis. Peaks, ratios, and slopes reported in the data were calculated as described by Hansen and others (2016).

Phytoplankton Enumeration: Phytoplankton enumeration was completed by BSA

Environmental Services in accordance with the American Public Health Association (APHA)

Standard Method 10200 (2012). This method focuses on counting and identifying microplankton (20-200 microns) and nanoplankton (2 to 20 microns). Briefly, phytoplankton were enumerated to the lowest possible taxonomic level using membrane-filtered slides, as described in McNabb (1960), and counted using a Leica DMLB compound microscope. The magnification used (100X or 1000X) depended upon the size of the dominant taxa and presence of particulates to ensure that enumeration and identification of taxa of various sizes was achieved. A minimum of 400 natural units (colonies, filaments, and unicells) or a minimum of 50 fields were counted from each sample. Counting 400 natural units provides accuracy within 90-percent confidence limits (Lund and others, 1958). In addition, an entire strip of the filter was counted at high magnification (usually 630X) along with one-half of the filter at a lower magnification (usually 400X) to ensure complete species reporting. Cell biovolume of all identified taxa were estimated using formulae for solid geometric shapes that most closely match the cell shape; method by Hillebrand and others (2002). Biovolume calculations were based on measurements of 10 organisms per taxon for each sample, where possible. For taxa with substantial size variation (such as diatoms), size classes were designated arbitrarily to determine average cell size (biovolume). For each taxon, 10 cells were measured from each size class (assuming that enough were available). From Burkholder and Wetzel (1989), the mean biovolume within each size class were used to calculate the total biovolume contributed by the taxon to its representative sample.

Picocyanobacteria direct counts: Picocyanobacteria were enumerated via epifluorescence microscopy as described by Murrell and Lores (2004) using a Leica microscope and 1000X total magnification. Phycoerythrin-containing cells (PE) and phycocyanin-containing cells (PC) were counted separately and then summed to obtain total picocyanobacteria densities for cells sized between 0.2 and 2.0 microns. For each sample, a minimum of 300 cells were counted unless this tally was not reached within 100 fields.

## Data processing

High resolution data from onboard continuous instruments not directly logged to the flow-through data collection system were merged based on timestamp to the nearest second. All data directly logged to the flow-through data collection system was processed using the pandas software library (0.220; McKinney, 2010) in Python (3.7.3, Van Rossum and Drake, 2009) to remove periods of compromised data (e.g. flow blockages, bubbles), to apply instrument corrections and unit conversions, and to apply a centered 20 second median filter to the time series. Final NO3 values were obtained by regressing instrument response against NO3 concentrations obtained from laboratory measurements of discrete samples collected through the course of the day. Individual sample results more than three standard deviations from the regression were judged to be outliers and removed from the regression (Pellerin and others, 2013)

Larger differences between SUNA and laboratory NO3 concentration data are assumed to arise from 1) actual spatial heterogeneity in environmental NO3 concentrations between water entering the flow through system and that captured in the sample bottle, 2) interferences of turbidity and dissolved organic matter using the UV absorption method for nitrate, or 3) errors in sample handling and laboratory analysis. Excepting these potential sources of error, the field and laboratory NO3 measurements are typically well correlated with a linear fit. For the simple linear regressions, the SUNA NO3 concentration data was the explanatory variable to compute the response variable of the laboratory NO3 concentration. P-values were <0.0001 for all regressions. Summary statistics and regressions for individual days are identified in Table 1.

Table 1. Summary statistics and linear regressions for nitrate (NO3) concentration data obtained from high-resolution SUNA instrument and discrete samples analyzed using standard laboratory methods [n, number of observations; µM, micromolar; mean, mean high-resolution nitrate concentration; median, median high-resolution nitrate concentration; min, minimum high-resolution nitrate concentration; max, maximum high-resolution nitrate concentration; SS, sum of squared error; SSE, sum of squared estimate of error; MSE, mean square error; RMSE, root mean square error; %, percent; MSPE, model standard percentage error; R2, coefficient of determination; *, asterisk indicates adjusted R2].

| Parameter | Date | n | min (µM) | max (µM) | median (µM) | mean (µM) | SS (µM) | SSE (µM) | MSE (µM) | RMSE (µM) | MSPE (%) | regression equation | p-value | R2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NO3 (SUNA) | 8/28/2019 | 5 | 9.4 | 16.6 | 13.6 | 13.6 | 29.7 | 0.0 | 9.9 | 0.1 | 1.3 | y = 0.9333x - 3.0176 | <0.0001 | 0.9969 |
| NO3 (SUNA) | 9/10/2019 | 11 | 7.2 | 15.5 | 12.7 | 11.5 | 116.7 | 4.5 | 0.5 | 0.9 | 11.8 | y = 0.8986x - 2.9658 | <0.0001 | 0.9178 |
| NO3 (SUNA) | 9/11/2019 | 8 | 4.8 | 16.4 | 8.1 | 9.7 | 114.6 | 3.0 | 0.5 | 1.1 | 14.8 | y = 0.9953x - 3.7078 | <0.0001 | 0.9365* |
| NO3 (SUNA) | 9/12/2019 | 10 | 7.3 | 15.5 | 13.9 | 12.7 | 79.8 | 2.1 | 10.0 | 0.7 | 17.8 | y = 1.057x - 6.2283 | <0.0001 | 0.9692* |

NH4 data was processed by first correcting the voltage output of the instrument for baseline drift based on the instrument response during periodic measurements of organic-free deionized water (n ≥10) and then using a regression model of the voltage response to a standard concentration. A regression coefficient of >0.97 R² was judged to be acceptable. The NH4 data timestamp was corrected to account for the time lag associated with the microfluidic processing prior to integration with other data sources.

Dissolved inorganic nitrogen (DIN) concentration was calculated as the sum of NO3, NO2 and NH4–both for the continuous data and the discrete sample data. Laboratory DON concentration was calculated by subtracting NH4, NO3, and NO2 data from the TDN data (i.e. DON = TDN – DIN). Continuous DOC concentrations were developed by regressing onboard fDOM data against laboratory determined DOC concentrations (R2 = 0.81) measured in discrete samples according to modified methods by Snyder and others (2018); these data are reported as estDOC. Due to a poor relation between fDOM and discrete sample DON concentrations (R2 = 0.21), we did not estimate continuous DON concentrations. Data exceeding three times the standard deviation of the residuals was judged to be an outlier and not included in the model.

Laboratory reported data for discrete samples from the National Water Quality Information System (NWIS; USGS, 2022) are included for convenience in a discrete data file. Accompanying laboratory reported data are NWIS qualifier and remark codes as they appear in NWIS (U.S. Geological Survey, 2011a, 2011b). For discrete nutrient sample concentrations, in addition to reporting nutrient parameters as they appear in NWIS (e.g., mg-N/L or mg-P/L), we also reported those same parameters converted to µM units. For laboratory reported values that are reported below the method lowest detection limit (LDL), half of the LDL is reported in the associated µM column (Clarke, 1998), and those data are used to bias correct the SUNA NO3 data (Pellerin and others, 2013) and to generate the spatially aligned datasets. Table 2 summarizes mg/L to µM conversion factors and ½ the LDL values (Clarke, 1998).

Table 2. Summary of conversions factors used to calculate micromolar nutrient concentrations, NWQL method lowest detection limit (LDL), and ½ LDLs used for bias correction and model calculations.

| Parameter | Conversion factor | LDL (mg-/L) | LDL (μM) | 1/2 LDL μM |
| --- | --- | --- | --- | --- |
| NO3 | 71.39 | 0.04 | 2.86 | 1.428 |
| NH4 | 71.39 | 0.01 | 0.71 | 0.36 |
| NO2 | 71.39 | 0.001 | 0.071 | 0.036 |
| TDN | 71.39 | 0.05 | 3.57 | 1.785 |
| PO4 | 32.28 | 0.004 | 0.13 | 0.065 |

### Spatial Alignment (SA)

Data were spatially aligned to geographic coordinate system (NAD 1983, 2011) to facilitate comparisons between dates and to minimize the effects of differential data density on the spatial interpolation calculations. The spatial alignment was performed using the GeoPandas software library in python (Jordahl, 2014), which assigned median-filtered data points to polygon shapefiles that are spaced at approximately 150 m intervals. Spatially aligned data were then aggregated by polygon shapefile to obtain summary statistics of all median-filtered data within a polygon.

Interpolated water quality data were created from the spatially aligned data using ESRI’s ArcGIS Pro Spline with Barriers tool (Terzopoulos and Witkin, 1988). The raster output was smoothed with the Focal Statistics tool, and interpolated values from the raster output were extracted to the spatially aligned points with the Extract Multi Values to Points tool to create continuous spatially aligned data across the mapping domain. Rasters were converted to vector format using the Contour tool, then clipped with the Clip tool using a Sacramento-San Joaquin Delta polygon obtained from the National Hydrography Dataset (U.S. Geological Survey, 2016). These data are referred to as spatially aligned (SA), and each data point is associated with a specific sampling day, latitude and longitude, and water quality parameter.

# References

American Public Health Association [APHA], 2012, Standard methods for the examination of water and wastewater, 22nd Edition: Washington, D.C., American Water Works Association, Water Environment Federation, 874 p.

Arar, E.J. and Collins, G.B., 1997, Method 445.0: In vitro determination of chlorophyll a and pheophytin a in marine and freshwater algae by fluorescence: Washington, D.C., U. S. Environmental Protection Agency, Office of Research and Development, National Exposure Research Laboratory. [Also available at https://cfpub.epa.gov/si/si_public_record_report.cfm?Lab=NERL&dirEntryId=309417.]

Beutler, M., Wiltshire, K., Meyer, B., Moldaenke, C., Lüring, C., Meyerhöfer, M., & Dau, H., 2002, A fluorometric method for the differentiation of algal populations in vivo and in situ: Photosynthesis Research, v. 72, p. 39-53,  https://doi.org/10.1023/A:1016026607048.

Burkholder, J.B. and Wetzel, R.G., 1989, Microbial colonization on natural and artificial macrophytes in a phosphorus-limited, Hardwater Lake: Journal of Phycology, v. 25, p. 55-65, https://doi.org/10.1111/j.0022-3646.1989.00055.x.

Carlson, R. M., 1978, Automated separation and conductimetric determination of ammonia and dissolved carbon dioxide: Analytical Chemistry, v. 50, p. 1528-1531, https://doi.org/10.1021/ac50033a035.

Clarke, J.U., 1998, Evaluation of censored data methods to allow statistical comparisons among very small samples with below detection limit observations: Environmental Science & Technology, v. 32, no. 1, p. 177-183, https://pubs.acs.org/doi/10.1021/es970521v.

Downing, B.D., Bergamaschi, B.A., Kendall, C., Kraus, T.E.C., Dennis, K.J., Carter, J.A. and Von Dessonneck, T.S., 2016, Using continuous underway isotope measurements to map water residence time in hydrodynamically complex tidal environments: Environmental Science & Technology, v. 50, no. 24, p. 13387-13396, https://doi.org/10.1021/acs.est.6b05745.

Fichot, C.G., Downing, B.D., Bergamaschi, B.A., Windham-Myers, L., Marvin-DiPasquale, M.,

Thompson, D.R. and Gierach, M.M., 2016. High-resolution remote sensing of water quality in

the San Francisco Bay–Delta Estuary: Environmental Science & Technology, v. 50, no. 2, p .573-583, https://doi.org/10.1021/acs.est.5b03518.

Fishman, M.J., 1993, Methods of analysis by the U.S. Geological Survey National Water Quality Laboratory- Determination of inorganic and organic constituents in water and fluvial sediments: U.S. Geological Survey Open-File Report 93-125, 217 p., https://doi.org/10.3133/ofr93125.

Hansen, A.M., Kraus, T.E.C., Pellerin, B. A., Fleck, J. A., Downing, B. D., Bergamaschi, B. A., 2016, Optical properties of dissolved organic Matter (DOM): effects of biological and photolytic degradation: Limnology and Oceanography, v. 61, p. 1015-1032, https://doi.org/10.1002/lno.10270.

Hansen, A. M., Fleck, J. A., Kraus, T. E. C., Downing, B. D., Von Dessonneck, T., Bergamaschi, B. A., 2018, Procedures for using the Horiba Scientific Aqualog® fluorometer to measure absorbance and fluorescence from dissolved organic matter: U. S. Geological Survey Open-File Report 2018-1096, 31 p., https://doi.org/10.3133/ofr20181096.

Hillebrand, H., Dürselen, C., Kirschtel, D., 2002, Biovolume calculation for pelagic and benthic microalgae: Journal of Phycology v. 35, p. 403-424, https://doi.org/10.1046/j.1529-8817.1999.3520403.x.

Jabusch, T., Bresnahan, P., Trowbridge, P., Novick, E., Wong, A., Salomon, M. and Senn, D., 2016, Summary and evaluation of Delta subregions for nutrient monitoring and assessment: Richmond, Calif. San Francisco Estuary Institute, Contribution No. 789, https://www.sfei.org/documents/delta-subregions.

Jordahl, K., 2014, GeoPandas: Python tools for geographic data, accessed December 2, 2020, at https://geopandas.org/en/stable/.

Kimmerer, W., Wilkerson, F., Downing, B., Dugdale, R., Gross, E.S., Kayfetz, K., Khanna, S., Parker, A.E. and Thompson, J., 2019, Effects of drought and the emergency drought barrier on the ecosystem of the California Delta: San Francisco Estuary and Watershed Science, v. 17, no. 3, https://doi.org/10.15447/sfews.2019v17iss3art2.

Lund, J. W. G., Kipling, C., Le Cren, E. D., 1958, The inverted microscope method of estimating algal numbers and the statistical basis of estimations by counting: Hydrobiologia, v. 11, p. 143-170. https://doi.org/10.1007/BF00007865.

McKinney, W., 2010, Data structures for statistical computing in python: Proceedings of the 9th Python in Science Conference, v. 445, p. 51–56, https://doi.org/10.25080/Majora-92bf1922-00a.

McNabb, C. D., 1960, Enumeration of freshwater phytoplankton concentrated on the membrane filter: Limnology and Oceanography, v. 5, p. 57-61, https://doi.org/10.4319/lo.1960.5.1.0057.

Murrell, M.C. and Lores, E.M., 2004, Phytoplankton and zooplankton seasonal dynamics in a subtropical estuary: importance of cyanobacteria: Journal of Plankton Research, v. 26, p. 371-382, https://doi.org/10.1093/plankt/fbh038.

Patton, C.J., Kryskalla. J.R., 2003, Methods of analysis by the U.S. Geological Survey National Water Quality Laboratory-Evaluation of alkaline persulfate digestion as an alternative to Kjeldahl digestion for determination of total and dissolved nitrogen and phosphorus in water: U.S. Geological Survey Water-Resources Investigations Report 2003-4174, 33 p., https://doi.org/10.3133/wri034174.

Patton, C.J. and Kryskalla, J.R., 2011, Colorimetric determination of nitrate plus nitrite in water by enzymatic reduction, automated discrete analyzer methods: U.S. Geological Survey Techniques and Methods 5B-8, 34 p., https://doi.org/10.3133/tm5B8.

Patton, C.J. and Truitt, E.P., 1992, Methods of analysis by the U.S. Geological Survey National Water Quality Laboratory-Determination of the total phosphorus by a Kjeldahl digestion method and an automated colorimetric finish that includes dialysis: US Geological Survey Open-File Report 92-146, 39 p., https://doi.org/10.3133/ofr92146.

Pellerin, B.A., Bergamaschi, B.A., Downing, B.D., Saraceno, J.F., Garrett, J.A., and Olsen, L.D., 2013, Optical techniques for the determination of nitrate in environmental waters: guidelines for instrument selection, operation, deployment, maintenance, quality assurance, and data reporting: U.S. Geological Survey Techniques and Methods 1–D5, 37 p., https://doi.org/10.3133/tm1D5.

Potter, B. and Wimsatt, J., 2012, USEPA method 415.3: quantifying TOC, DOC, and SUVA: Journal American Water Works Association, v. 104, 6, p. E358-E369, https://doi.org/10.5942/jawwa.2012.104.0086.

Snyder, L., Potter, J.D., and McDowell, W.H., 2018. An evaluation of nitrate, fDOM, and turbidity sensors in New Hampshire streams: Water Resources Research, v. 54, no. 3, p. 2466-2479, https://doi.org/10.1002/2017WR020678.

Stumpner, E. B., Bergamaschi, B. A., Kraus, T. E. C., Parker, A. E., Wilkerson, F. P., Downing, B. D., Dugdale, R. C. Murrell, M. C., Carpenter, K. D., Orlando, J. L., and Kendall, C., 2020, Spatial variability of phytoplankton in a shallow tidal freshwater system reveals complex controls on abundance and community structure: Science of The Total Environment, v. 700, no. 134392, 17 p., https://doi.org/10.1016/j.scitotenv.2019.134392.

Terzopoulos, D. and Witkin, A., 1988, Physically based models with rigid and deformable components: IEEE Computer Graphics and Applications, v. 8, no. 6, p. 41-51, https://doi.org/10.1109/38.20317.

Thompson, L., Mussen, T.D., Cook, M., Nordin, J., Noss, J., Bigler, U., Ramamoorthy, S., Berg, G.M., Driscoll, S., Herrmann, C., Kimmerer, W., Ignoffo, T., Kraus, T.E.C., Fackrell, J., Bergamaschi, B., Guerin, M., and Rachiele, R., 2023, Sacramento River nutrient change study. A report to the Delta Regional Monitoring Program, State Water Contractors, and U.S. Bureau of Reclamation, https://deltarmp.org/Documents/Sacramento_River_Nutrient_Change_Study.pdf.

U.S. Geological Survey, 2016, National Hydrography Dataset (NHD): U. S. Geological Survey, National Geospatial Technical Operations Center (NGTOC), accessed December 14, 2018, at http://nhd.usgs.gov.

U.S. Geological Survey, 2011a, Value Qualifier Codes: U.S. Geological Survey National Water Information System: Help System, https://help.waterdata.usgs.gov/code/val_qual_cd_query.

U.S. Geological Survey, 2011b, Remark Codes: U.S. Geological Survey, National Water Information System: Help System, https://help.waterdata.usgs.gov/remark_cd.

U.S. Geological Survey, 2022, USGS water data for the Nation: U.S. Geological Survey, National

Water Information System database, http://dx.doi.org/10.5066/F7P55KJN.

Van Rossum, G., and Drake, F. L., 2009, Python 3 Reference Manual. Scotts Valley, CA: CreateSpace, accessed March 15, 2017.