Assessing spatial variability of nutrients, phytoplankton, and related water quality constituents in the California Sacramento-San Joaquin Delta at the landscape scale: 2023 High-resolution mapping surveys

O'Donnell, K., Uebner, M.Q., Brinkman, J, Richardson, E.T., Soto Perez, J., Sturgeon, C.L., Maestas, A., Ayers, A., Nakatsuka, K., Bouma Gregson, K., Kraus, T.E.C., and Bergamaschi, B., 2025, Assessing spatial variability of nutrients, phytoplankton, and related water quality constituents in the California Sacramento-San Joaquin Delta at the landscape scale: 2023 High-resolution mapping surveys: U.S. Geological Survey data release, https://doi.org/10.5066/P9TW0LT5.

# Methods

Broad-area, high-resolution, boat-based water quality mapping surveys of the Sacramento-San Joaquin Delta (Delta) and Suisun Bay were conducted in May, July, and October of 2023. The surveys in May and July were comprised of four successive days of high-resolution mapping. The regions covered included the north-south axis of the Delta from the Sacramento River north of Freeport to the Harvey Banks export facility located in the southern Delta, including Old and Middle Rivers; the east-west axis of the Delta from the Sacramento River at Rio Vista to the San Joaquin River at Stockton, including Disappointment Slough and the North and South Forks of the Mokelumne River; and the northern and western Delta from the northern reaches of the Cache Slough Complex to the Grizzly, Honker, and Suisun Bays. On each day, the route of the mapping survey was determined by weather, tides and other logistical considerations. Data were collected at high speed to minimize the extent to which tidal currents obscure nutrient and phytoplankton distributions.

The survey in October was comprised of two successive days of high-resolution mapping. The regions covered in October included the north-south axis of the Delta from the Sacramento River north of Freeport to the Harvey Banks export facility located in the southern Delta, including Old and Middle Rivers; as well as the east-west axis of the Delta from the Sacramento River at Rio Vista and Franks Tract in the east to Grizzly, Honker, and Suisun Bays in the west. October surveys were intended to be four consecutive days, but were cut short due to a boat engine failure. On November 1st and 2nd, discrete water quality samples were collected from the regions not covered with high-resolution mapping. In November at some sampling locations, in situ water quality measurements were taken with a multiparameter water quality sonde (YSI EXO2; Xylem Inc. (EXO), Rye Brook, New York). These supplementary data are included in the discrete data file (DeltaMapping_2023_DS.txt).

## Sampling and analysis

### Underway high resolution (HR) water sampling and onboard analyses

Surveys were conducted during daylight hours from a moving boat, the USGS research vessel (R/V) George Aiken, on May 15th through 18th, July 17th through 20th, and October 23rd and 24th, 2023, following the approach described by Bergamaschi and others (2024), Richardson and others (2023b), and Stumpner and others (2020). For analysis, sample water was continuously pumped onto the boat using two SHURflo Aqua King II pumps at a pressure of 60 psi and rate of 5 GPM (SHURflo Pentair, London, United Kingdom) while underway at speeds up to 30 miles per hour (mph) (13 meters per second [m/s]) using a pick-up tube mounted at a fixed depth of approximately 1 m below the surface.  Water was routed through a 178 micron (um) in-line strainer (Cole Parmer; EW-29595-47, Vernon Hills, Illinois) to remove large debris. Sample water was split into separate flow paths as described below, and each flow path was routed through a pressure-compensated manifold that maintained system pressure at a prescribed level (~20 psi) irrespective of boat speed (Bergamaschi and others, 2024). A 2-stage debubbler was used to remove bubbles that can interfere with instrument performance.  Flow rates were maintained by metering the discharge line, except for the flow path serving the open split interface, which was metered on the feed line. Flows were continuously monitored using a sight gauge (0.1-1 GPM Flow Meter LZM-15 with adjustable knob) for each flow path.

A manifold delivered unfiltered water to the following flow paths: (1) A flow-through system consisting of a thermosalinograph that recorded temperature, conductance, and salinity (Sea-Bird Scientific SB45 (TSG), Bellevue, Washington); fluorometers that measured fluorescence of chlorophyll (fCHL) and fluorescence of dissolved organic matter (fDOM, Sea-Bird Scientific WETstar (WS), Philomath, Oregon); a beam transmissometer that recorded transmittance (Sea-Bird Scientific model C-Star transmissometer (CStar), Philomath, Oregon); a nitrate (NO3) analyzer (SUNA V2; Sea-Bird Scientific, Bellevue, Washington). (2) A second path provides water to a multiparameter water quality sonde (YSI EXO2) equipped with sensors to measure temperature, specific conductance (SpC), turbidity, pH, dissolved oxygen (DO), fDOM and fCHL. (3) A third path passed water to a fluorometer designed to measure different algal classes (FluoroProbe III (FP); BBE Moldaenke, Schwentinental, Germany; Eijkelkamp, 2014; Beutler and others, 2002).

In addition to water measurements, air temperature was also collected. A Campbell Scientific 108 Air Temperature sensor (Campbell Scientific, Logan, Utah) was mounted under the roof at the stern of the boat near the manifold system, 2-3 m above the surface of the water. The heat shield is not used, but it is shaded by the roof to protect from direct sunlight, and close to the cabin of the boat to protect from wind.

All instrumentation was cleaned, and calibrations were checked prior to each use following the manufacturer's recommendation. Data for most instruments were recorded at 1-second frequency on a single data logger (CR6, Campbell Scientific, Logan, Utah) together with a timestamp and boat position obtained from a high-resolution global positioning system receiver (16X-HVS, Garmin, Olathe, Kansas). The FluoroProbe logged data internally and to the host software every 1-2 seconds. All data were displayed in real time, so scientists on board could respond when they observed changes relevant to study objectives, or if issues with flow or instrument performance were noted. Data collected in situ are referred to as high resolution (HR) parameters.

### Discrete water sampling (DS) and laboratory analyses

Discrete water samples (DS) were collected either while underway or–in the case of more extensive sample requirements–while stopped on station. Discrete samples were collected while underway by passing water from the pickup tube through a 0.2-micron filter into a sample bottle. At designated sampling stations (approximately 10 per day) samples for determination of ammonium (NH4), nitrite (NO2), nitrate (NO3), phosphate (PO4), dissolved organic carbon (DOC), total dissolved nitrogen (TDN), and total dissolved phosphorus (TDP) concentrations were collected using this system. Additionally, unfiltered samples were collected approximately from 1 m depth using a submersible pump for analysis of chlorophyll a (Chl a), phaeophytin, particulate carbon (PC), and particulate nitrogen (PN) concentrations, and suspended sediment concentration (SSC).

Most samples were stored on wet ice in the dark during transit to the laboratory where they were stored at 2° Celsius (C). Dissolved organic carbon samples were preserved to <2 pH with high-purity sulfuric acid the day following collection and stored at 2°C. Samples for pigment analysis were filtered within 24 hours of collection and immediately frozen (-80°C). Chl a samples were filtered through a 0.7 um nominal pore size glass fiber filter (Advantec MFS, Inc, Dublin, California). Chl a associated with the larger sized cell fraction were filtered through a 5 um pore size 25 mm polycarbonate filter (General Electric Healthcare, Chicago, Illinois). Samples for determination of suspended sediment concentration (SSC), particulate carbon (PC), and particulate nitrogen (PN) were filtered through a pre-weighed 0.7 um nominal pore size combusted glass fiber filter. Frozen Chl a and SSC/PCPN filters and chilled nutrient were shipped to the Chesapeake Biological Laboratory (Solomons, Maryland).

Concentrations of nutrients were determined colorimetrically using an automated discrete photometric analyzer. Concentration of nitrogen as nitrite (NO2-N) was determined by colorimetric analysis after diazotization with sulfanilamide (USEPA, 1993a).  For nitrate plus nitrite (NO3-N + NO2-N), samples were analyzed using enzymatic reduction (ASTM, 2014), and if results were below the reporting limit of 0.15 mg/L, they were rerun using cadmium reduction (USEPA, 1993a) due to the lower detection limit of that method. Concentration of ammonium as nitrogen (NH4-N) was determined by colorimetric analysis after reaction with alkaline phenol-hypochlorite (Rice and others, 2017). Orthophosphate as phosphorus (PO4-P; also referred to as soluble reactive phosphorus) was determined by colorimetric analysis after reaction with ammonium molybdate and reduction with ascorbic acid (USEPA, 1993b). Total dissolved nitrogen (TDN) and total dissolved phosphorus (TDP) were determined by alkaline persulfate digestion (USEPA 1993a, 1993b).

For suspended sediment concentration (SSC), particulate carbon (PC), and particulate nitrogen (PN) analyses, filters were first dried at approximately 105°C to a constant weight. Once dry, filters were re-weighed, and compared to the weight recorded prior to filtering in order to compute SSC (Kopp, 1979). Then, a weighed amount of dried particulate was combusted at high temperature and measured using an elemental analyzer to determine PC and PN concentrations (Zimmerman and others, 1997). Chlorophyll a and phaeophytin concentrations were determined fluorometrically according to EPA Method 445.0 (Arar and Collins, 1997), using mechanical grinding to macerate cells.

Dissolved organic carbon concentrations were measured using a total organic carbon analyzer (TOC-VCSH, Shimadzu Scientific Instruments, Columbia, Maryland) using high-temperature catalytic combustion according to a modified version of USEPA method 415.3 (Potter and Wimsatt, 2012). Absorbance spectra and fluorescence EEMs were simultaneously measured using an Aqualog® equipped with a photodiode and charge-coupled device (CCD) (Horiba Instruments, Piscataway, New Jersey) according to the method described by Hansen and others (2018). Briefly, excitation and absorbance spectra were measured using a 150 W xenon lamp, a 5 nm bandpass, and a 1 s integration time at wavelengths of 240–600 nm. Emission spectra were collected with a CCD at approximately 1.64 nm (4 pixel) intervals at wavelengths of 250–600 nm. To reduce ultraviolet light exposure of the sample and limit the effects of photobleaching, excitation and absorbance wavelengths were scanned from low to high energy (i.e., visible to ultraviolet range) during analysis. Peaks, ratios, and slopes reported in the data were calculated as described by Hansen and others (2016).

Phytoplankton enumeration was completed by BSA Environmental Services (Beachwood, Ohio) in accordance with the American Public Health Association (APHA) Standard Method 10200 (2023). Picophytoplankton were enumerated via epifluorescence microscopy as described by Murrell and Lores (2004). Phytoplankton and picophytoplankton enumeration data will be published in a USGS Data Release that is periodically revised, which can be found at https://doi.org/10.5066/P97ZBPLH (Richardson and others, 2023a).

## Data processing

High-resolution (HR) SUNA NO3 data were corrected to discrete sample data. For each month, discrete NO3 concentration was subtracted from NO3 measured by the SUNA. The median difference for a given month was then subtracted from all raw SUNA NO3 values for a given month. Possible differences between SUNA and laboratory NO3 concentration data are assumed to arise from 1) actual spatial heterogeneity in environmental NO3 concentrations between water entering the flow through system and that captured in the sample bottle; 2) interferences of turbidity and dissolved organic matter using the UV absorption method for nitrate; or 3) errors in sample handling and laboratory analysis. For individual offset equations, please refer to the “DeltaMapping_2023_DataDictionary_CorrectionsAndOffsets.csv” file within the data dictionary (DeltaMapping_2023_DataDictionary.zip).

All data were processed using the pandas software library (0.220; McKinney, 2010) in Python (3.7.3; Van Rossum and Drake, 2009) to remove periods of compromised data (e.g. flow blockages, bubbles), to apply instrument corrections and unit conversions, and to apply a centered 20 second median filter to the time series.

Continuous DOC concentrations were developed by regressing onboard fDOM data against laboratory determined DOC concentrations measured in discrete samples (Snyder and others, 2018). These data are reported as estDOC. Please refer to the “DeltaMapping_2023_DataDictionary_CorrectionsAndOffsets.csv” file within the data dictionary (DeltaMapping_2023_DataDictionary.zip) for individual regression equations. Data exceeding 3 times the standard deviation of the residuals was judged to be an outlier and not included in the model.

Laboratory reported data for discrete samples (DS) from Water Data for the Nation (WDFN; USGS, 2025) are included for convenience in a discrete data file (DeltaMapping_2023_DS.txt). Remark codes, as they appear on WDFN, are included in the DS file (U.S. Geological Survey, 2011). For discrete nutrient sample concentrations, in addition to reporting nutrient parameters as they appear in WDFN (e.g., mg-N/L [milligrams nitrogen per liter]), we also reported those same parameters converted to uM (micromolar) units. For laboratory reported values that are reported below the method lowest detection limit (LDL), half of the LDL is reported in the associated uM column (Clarke, 1998). These uM data were used to bias correct the SUNA NO3 data (Pellerin and others, 2013). Table 1 summarizes mg/L to uM conversion factors and 1/2 the LDL values (Clarke, 1998).

Table 1. Summary of conversions factors used to calculate micromolar nutrient concentrations, CBL method detection limit (MDL), and 1/2 LDLs used for bias correction and model calculations.

| Parameter | Conversion factor | MDL (mg-/L) | MDL (uM) | 1/2 MDL uM |
| --- | --- | --- | --- | --- |
| NO3 | 71.39 | 0.0009 | 0.064 | 0.032 |
| NH4 | 71.39 | 0.009 | 0.64 | 0.643 |
| NO2 | 71.39 | 0.0009 | 0.064 | 0.032 |
| TDN | 71.39 | 0.05 | 3.57 | 1.785 |
| PO4 | 32.28 | 0.0034 | 0.11 | 0.055 |

### Definition of regions and reaches for analysis

Aside from evaluating the Delta as a whole, it is useful to divide the survey data into smaller sub-sets for analysis. We chose two main approaches to do this. First, we defined nine Study Regions based on hydrodynamically distinct zones identified by Jabusch and others (2016). Broadly speaking, the Delta has three tidal transition zones (north Delta, Mokelumne system, south Delta), two strongly tidally forced tidal lagoons (western and central Delta), two dead-end channel systems (CSC [Cache Slough Complex], and Suisun Marsh, an area we define as a complex of tidal and managed wetlands near Ryer Island), and the brackish part of the estuary that connects the Delta to San Francisco Bay (Suisun Bay). The nine regions defined in this study are (1) north Delta tidal transition zone, (2) Mokelumne tidal transition zone, (3) south Delta tidal transition zone, (4) central Delta tidal transition zone, (5) San Joaquin tidal transition zone, (6) western Delta tidal transition zone, (7) Cache Slough Complex, (8) Suisun Bay, and (9) Suisun Marsh channel system. A ‘Transition Zone’ is where transport transitions from being mostly unidirectional, like in rivers, to mostly tidal. These are similar to regions proposed by Jabusch and others (2016) to evaluate data collection efforts and nutrient status and trends analyses in the Delta. Their regions were based on Operational Landscape Units (OLUs) originally developed to represent restoration opportunity areas based on an understanding of ecological function, physical drivers, existing constraints, and elevation gradients, while taking into account hydrologic features, watershed boundaries, and DSM2 (Delta Simulation Model II) modeling requirements (Grenier and Grossinger, 2016). Second, we associated data with the geographic name of each water feature (e.g., Sacramento River, Georgiana Slough, Old River) based on widely recognized naming conventions that were cross referenced with the USGS Geographic Names Information System (GNIS) database (USGS, 2022).

# References

American Public Health Association (APHA), American Water Works Association, Water Environment Federation, Lipps W.C., Braun-Howland E.B., Baxter T.E., eds., 2023, Standard Methods for the Examination of Water and Wastewater, 24th ed, Washington DC: APHA Press.

Arar, E.J. and Collins, G.B., 1997, Method 445.0: In vitro determination of chlorophyll a and pheophytin a in marine and freshwater algae by fluorescence: Washington, D.C., U. S. Environmental Protection Agency, Office of Research and Development, National Exposure Research Laboratory. [Also available at https://cfpub.epa.gov/si/si_public_record_report.cfm?Lab=NERL&dirEntryId=309417.]

ASTM Standard D7781-14, 2014, “Standard Test Method for Nitrite-Nitrate in Water by Nitrate Reductase,” ASTM International, West Conshohocken, PA.

Bergamaschi, B.A., Kraus, T.E.C., Downing, B.D., Stumpner, E.B., O’Donnell, K., Hansen, J.A., Soto Perez, J., Richardson, E.T., Hansen, A.M., and Gelber, A., 2024, Assessing spatial variability of nutrients, phytoplankton, and related water-quality constituents in the California Sacramento–San Joaquin Delta at the landscape scale—2018 high resolution mapping surveys: U.S. Geological Survey Scientific Investigations Report 2023–5060, 47 p., https://doi.org/10.3133/sir20235060.

Beutler, M., Wiltshire, K., Meyer, B., Moldaenke, C., Lüring, C., Meyerhöfer, M., & Dau, H., 2002, A fluorometric method for the differentiation of algal populations in vivo and in situ: Photosynthesis Research, v. 72, p. 39-53,  https://doi.org/10.1023/A:1016026607048.

Burkholder, J.B. and Wetzel, R.G., 1989, Microbial colonization on natural and artificial macrophytes in a phosphorus-limited, Hardwater Lake: Journal of Phycology, v. 25, p. 55-65, https://doi.org/10.1111/j.0022-3646.1989.00055.x.

Carlson, R. M., 1978, Automated separation and conductimetric determination of ammonia and dissolved carbon dioxide: Analytical Chemistry, v. 50, p. 1528-1531, https://doi.org/10.1021/ac50033a035.

Clarke, J.U., 1998, Evaluation of censored data methods to allow statistical comparisons among very small samples with below detection limit observations: Environmental Science & Technology, v. 32, no. 1, p. 177-183, https://pubs.acs.org/doi/10.1021/es970521v.

Downing, B.D., Bergamaschi, B.A., Kendall, C., Kraus, T.E.C., Dennis, K.J., Carter, J.A. and Von Dessonneck, T.S., 2016, Using continuous underway isotope measurements to map water residence time in hydrodynamically complex tidal environments: Environmental Science & Technology, v. 50, no. 24, p. 13387-13396, https://doi.org/10.1021/acs.est.6b05745.

Eijkelkamp, 2014. FluoroProbe User Manual. V. E2, accessed March 15, 2017.

Fishman, M.J., 1993, Methods of analysis by the U.S. Geological Survey National Water Quality Laboratory- Determination of inorganic and organic constituents in water and fluvial sediments: U.S. Geological Survey Open-File Report 93-125, 217 p., https://doi.org/10.3133/ofr93125.

Grenier, L., Grossinger, R., 2016, Tools for landscape-scale restoration planning in the Delta. Presentation to the Delta Independent Science Board, January 16, 2013, 2nd Floor Conference Center, Sacramento, CA.

Hansen, A.M., Kraus, T.E.C., Pellerin, B. A., Fleck, J. A., Downing, B. D., Bergamaschi, B. A., 2016, Optical properties of dissolved organic Matter (DOM): effects of biological and photolytic degradation: Limnology and Oceanography, v. 61, p. 1015-1032, https://doi.org/10.1002/lno.10270.

Hansen, A. M., Fleck, J. A., Kraus, T. E. C., Downing, B. D., Von Dessonneck, T., Bergamaschi, B. A., 2018, Procedures for using the Horiba Scientific Aqualog® fluorometer to measure absorbance and fluorescence from dissolved organic matter: U. S. Geological Survey Open-File Report 2018-1096, 31 p., https://doi.org/10.3133/ofr20181096.

Hillebrand, H., Dürselen, C., Kirschtel, D., Pollingher, U., and Zohary, T., 1999, Biovolume calculation for pelagic and benthic microalgae: Journal of Phycology v. 35, p. 403-424, https://doi.org/10.1046/j.1529-8817.1999.3520403.x.

Kopp, J.F., 1979, Method No. 160.2 in Methods for chemical analysis of water and wastes, Environmental Monitoring and Support Laboratory, Office of Research and Development, US Environmental Protection Agency, Report No. EPA-600/4-79-020.

Lund, J. W. G., Kipling, C., Le Cren, E. D., 1958, The inverted microscope method of estimating algal numbers and the statistical basis of estimations by counting: Hydrobiologia, v. 11, p. 143-170. https://doi.org/10.1007/BF00007865.

McKinney, W., 2010, Data structures for statistical computing in python: Proceedings of the 9th Python in Science Conference, v. 445, p. 51–56, https://doi.org/10.25080/Majora-92bf1922-00a.

McNabb, C. D., 1960, Enumeration of freshwater phytoplankton concentrated on the membrane filter: Limnology and Oceanography, v. 5, p. 57-61, https://doi.org/10.4319/lo.1960.5.1.0057.

Murrell, M.C. and Lores, E.M., 2004, Phytoplankton and zooplankton seasonal dynamics in a subtropical estuary: importance of cyanobacteria: Journal of Plankton Research, v. 26, p. 371-382, https://doi.org/10.1093/plankt/fbh038.

Pellerin, B.A., Bergamaschi, B.A., Downing, B.D., Saraceno, J.F., Garrett, J.A., and Olsen, L.D., 2013, Optical techniques for the determination of nitrate in environmental waters: guidelines for instrument selection, operation, deployment, maintenance, quality assurance, and data reporting: U.S. Geological Survey Techniques and Methods 1–D5, 37 p., https://doi.org/10.3133/tm1D5.

Potter, B. and Wimsatt, J., 2012, USEPA method 415.3: quantifying TOC, DOC, and SUVA: Journal American Water Works Association, v. 104, 6, p. E358-E369, https://doi.org/10.5942/jawwa.2012.104.0086.

Rice, E., Baird, R., and Eaton, A., 2017, 4500-NH3 nitrogen (ammonia) G: Standard methods for examination of water and wastewater. American Public Health Association, American Water Works Association, and Water Environment Federation, Washington, DC.

Richardson, E.T., Bouma-Gregson, K., Kraus, T.E.C., O'Donnell, K., Sturgeon, C.L., Soto Perez, J., Delascagigas, A., Nakatsuka, K.K., Burau, D.J., Gelber, A.D., Von Hoyningen Huene, B.L., Jumps, N.I., and Bergamaschi, B.A., 2023a, Phytoplankton Species Composition and Abundance in the Sacramento-San Joaquin River Delta: Microscopic Enumeration of USGS Samples, beginning in 2016 (ver. 1.1, December 2023): U.S. Geological Survey data release, https://doi.org/10.5066/P97ZBPLH.

Richardson, E.T., Hansen, A.M., Kraus, T.E., Downing, B.D., Forsberg, D., Stillian, J., O’Donnell, K., Sturgeon, C.L., and Bergamaschi, B.A., 2023b, A novel boat‐based field application of a high‐frequency conductometric ammonium analyzer to characterize spatial variation in aquatic ecosystems: Limnology and Oceanography: Methods, https://doi.org/10.1002/lom3.10579.

Snyder, L., Potter, J.D., and McDowell, W.H., 2018. An evaluation of nitrate, fDOM, and turbidity sensors in New Hampshire streams: Water Resources Research, v. 54, no. 3, p. 2466-2479, https://doi.org/10.1002/2017WR020678.

Stumpner, E.B., Bergamaschi, B.A., Kraus, T.E.C., Parker, A.E., Wilkerson, F.P., Downing, B.D., Dugdale, R.C. Murrell, M.C., Carpenter, K.D., Orlando, J.L., and Kendall, C., 2020, Spatial variability of phytoplankton in a shallow tidal freshwater system reveals complex controls on abundance and community structure: Science of The Total Environment, v. 700, no. 134392, 17 p., https://doi.org/10.1016/j.scitotenv.2019.134392.

USEPA, 1993a, Method 353.2: Determination of nitrate‐nitrite nitrogen by automated colorimetry (Report Number EPA/600/R-93/100): Environ. Monitoring Systems Lab.

USEPA, 1993b, Method 365.1: Determination of Phosphorus by Semi-Automated Colorimetry (Report Number EPA/600/R-93/100): Methods for chemical analysis of water and wastes.

U.S. Geological Survey, 2011, Remark Codes: U.S. Geological Survey, National Water Information System: Help System, accessed September 16, 2024, at https://help.waterdata.usgs.gov/remark_cd.

U.S. Geological Survey, 2022, Geographic names information system (GNIS) website: U.S. Geological Survey, accessed June 19, 2023, at https://www.usgs.gov/tools/geographic-names-information-system-gnis.

U.S. Geological Survey, 2025, USGS water data for the Nation: U.S. Geological Survey, National Water Information System database, accessed May 30, 2025, at https://doi.org/10.5066/F7P55KJN.

Van Rossum, G., and Drake, F. L., 2009, Python 3 Reference Manual. Scotts Valley, CA: CreateSpace, accessed March 15, 2017.

Zimmermann, C.F., Keefe, C.W., and Bashe, J., 1997, Method 440.0: Determination of Carbon and Nitrogen in Sediments and Particulates of Estuarine/Coastal Waters Using Elemental Analysis: United States Environmental Protection Agency, Office of Research and Development, National Exposure Research Laboratory.