###### Methods

Field sampling by boat in Franks Tract occurred on July 14, August 9, and November 3, 2022. During each sampling event personnel collected handheld hyperspectral radiance and water quality data. The sampling method was designed to collect field data that would be comparable with satellite remote sensed data from Ocean Land Color Instrument (OLCI) on the Sentinel-3 A and B satellites. The hyperspectral field data were used to calculate non-dimensional surface reflectance (Rhos, ρs), which can be compared to the reflectance values from the OLCI sensor. The water quality data were used to evaluate how different water quality parameters influence the shape of the ρs spectra.

Table 1. Remote sensing algorithms calculated for Franks Tract. The first row shows the general equation, and subsequent rows identify the wavelengths that are used for each algorithm. The Rλ± variable in the general equation represents the SVC (Spectra Vista Corporation) 512i spectroradiometer (Poughkeepsie, New York, USA) or OLCI ρs reflectance value at that wavelength.

| Algorithm | Reference |
| --- | --- |
| General equation for spectral shape algorithms: SSλ=Rλ- Rλ-+Rλ-- Rλ+× λ-λ-λ+-λ- | (Stumpf et al., 2016) |
| Cyanobacterial Index (CI) CI=SS681× -1 λ– = 665, λ = 681, λ+ = 709 | (Wynne et al., 2008) |
| SS(665) λ– = 620, λ = 665, λ+ = 681 | (Matthews et al., 2012; Lunetta et al., 2015) |
| CIcyano CIcyano= CI, SS665>00, &otherwise | (Lunetta et al., 2015) |

OLCI pixels are approximately 300 x 300 m in size and are in approximately the same location for every image. To account for spatial heterogeneity of water conditions within a pixel, field data were collected from 2–3 sites within most pixels. Prior to field-sampling, the centroids of an OLCI pixel were obtained and loaded into the boat’s navigation computer. The boat then traveled near the centroid of a pixel and dropped an anchor to prevent drifting from wind or waves. After data were collected from the site, the boat motored to 1-2 additional sites within the pixel, dropped anchor again, and collected data. A total of 16 sites within 7 pixels (P1–P7) were sampled across the three days, with 3 sites within P1 and P2 on July 14; 3 sites in P3, 2 sites in P4, and 1 site P5 on August 3; and 3 sites in P6 and 2 sites in P7 on November 3 (Table 2).

Remotely sensed data

Field-based hyperspectral data were collected with a Spectra Vista Corporation (SVC) 512i spectroradiometer (Poughkeepsie, New York, USA). The instrument collects radiance (W m-2 nm-1 sr-1) approximately every 3 nm from 350–1050 nm. Radiance data were aggregated into 5 nm bins by averaging the values in each bin. First, remote sensing reflectance (Rrs) was calculated, then non-dimensional surface reflectance (ρs) was calculated from the Rrs value. Rrs was calculated using an above-water measurement approach (Mobley, 1999; Ruddick et al., 2019b) with a 10% calibrated Spectralon reflectance plate (Labsphere, North Sutton, New Hampshire, USA). Radiance data were collected from the water, sky, and reflectance plate at an angle of approximately 40° from nadir and an azimuth of 135° from the sun. Rrs was calculated from calibration plate, water, and sky radiance values following equation 6 from Mobley (1999):

Rrs = (water – ρ × sky) / (ϖ × plate ÷ ρp) (sr-1).

Where ρ is 0.028 and represents the fraction of light that light that is reflected off the water’s surface (Mobley, 1999; Ruddick et al., 2019b). The product of ρ and the sky radiance is subtracted from the water radiance to calculate the water leaving radiance, which is a measure of the light that entered the water and then was scattered back out of the water, rather than light reflected off the surface. The denominator estimates total downward irradiance (Ruddick et al., 2019a), and the plate reflectance (ρp) per nm is calibrated by the manufacturer. Rrs was multiplied by ϖ to calculate the non-dimensional surface reflectance (ρs, unitless) (Mobley et al., 2016).

Triplicate sets of water, sky, and plate measurements were made at each site. During each set ten measurements of each water, sky, and plate target were collected, which were then averaged together into a single value for each replicate. The SVC instrument was set to average ten readings per saved measurement value, auto-calibrate the integration time, and recalibrate to a dark spectrum measurement for every reading.

Multi-spectral satellite data from the OLCI sensor on Sentinel-3 A and B was provided by NOAA National Center for Coastal Observing Science (NCCOS) and the NCCOS level 4 product, Rayleigh-corrected top-of-the-atmosphere non-dimensional surface reflectance (Rhos, ρs) used for algorithm calculations (Wynne et al., 2020). The OLCI bands used for the algorithms are 7.5–10 nm wide and centered on 620, 665, 681, 709, 754 nm (https://sentinels.copernicus.eu/web/sentinel/user-guides/sentinel-3-olci/resolutions/radiometric). Additionally, the level 4 pixel masks for land, clouds, mixed pixels, and adjacency pixels (Wynne et al., 2020) were also provided by NCCOS. Mixed pixels are at the edge of land and include both land and water are within the pixel. Adjacency pixels are full-water pixels that are close to land, therefore stray light from the adjacent land may enter the pixel and contaminate the water spectra for that pixel.

Water quality data

At each sampling site a suite of water quality measurements was collected to study how different parameters influence the optical properties of surface reflectance spectra. Discrete samples were collected for chl-a and dissolved organic carbon (DOC) while stopped at each site. DOC samples were collected by passing water from the pickup tube through a 0.2-micron filter into a sample bottle, stored chilled, and analyzed at the USGS Organic Matter Research Laboratory. In preparation for analysis, dissolved organic carbon samples were preserved to <2 pH with high-purity sulfuric acid the day following collection and stored at 2°C.

Unfiltered chl-a samples were collected approximately at 0.5-1 m depth using a submersible pump, then stored chilled. Total Chl-a samples were filtered through a 0.7 um nominal pore size glass fiber filter (Advantec MFS, Inc, Dublin, California) and analyzed at the USGS National Water Quality Laboratory (NWQL) in Denver, Colorado. Chl-a concentrations were determined fluorometrically according to EPA Method 445.0 (Arar and Collins, 1997). The 0.7 um filtered samples were extracted using sonication.

Dissolved organic carbon concentrations were measured using a total organic carbon analyzer (TOC-VCSH, Shimadzu Scientific Instruments, Columbia, Maryland) using high-temperature catalytic combustion according to a modified version of USEPA method 415.3 (Potter and Wimsatt, 2012). Absorbance spectra and fluorescence EEMs were simultaneously measured using an Aqualog® equipped with a photodiode and charge-coupled device (CCD) (Horiba Instruments, NJ, USA) according to the method described by Hansen and others (2018). Briefly, excitation and absorbance spectra were measured using a 150 W xenon lamp, a 5 nm bandpass, and a 1 s integration time at wavelengths of 240–600 nm. Emission spectra were collected with a CCD at approximately 1.64 nm (4 pixel) intervals at wavelengths of 250–600 nm. To reduce ultraviolet light exposure of the sample and limit the effects of photobleaching, excitation and absorbance wavelengths were scanned from low to high energy (i.e., visible to ultraviolet range) during analysis. Peaks, ratios, and slopes reported in the data were calculated as described by Hansen and others (2016).

Additionally, in situ water quality data, were collected at 0.5 and 1 m depths using a YSI sonde (YSI EXO2; Xylem Inc. (EXO), Rye Brook, New York). The sonde was lowered by hand to the approximate depth and data were collected for approximately 4 minutes at each depth. Parameters measured are water temperature (Celsius), conductivity (uS/cm), specific conductivity (uS/cm), salinity (psu), pH (standard units), fluorescent chlorophyll (fCHL; ug/L), blue-green algae fluorescence (BGA; RFU), fluorescence of dissolved organic matter (fDOM,; QSU), pressure (psi), dissolved oxygen (DO; mg/L), and turbidity (FNU).

Remote sensing cyanobacterial abundance indices

Field and satellite Rrs data were used in three algorithms to estimate cyanobacterial biomass. The CI, CIcyano, and SS(665) algorithms were calculated from SVC or OLCI ρs data according to equations in Table 1. Because OLCI bandwidths are wider than the SVC bands, three SVC 5 nm bins were averaged together to match the widths of the OLCI wavelengths included in each OLCI band. Pearson correlations between remote sensing and water sample parameters were calculated by averaging all replicate measurements within a site and then comparing site averaged values among parameters.

Table 2.

Site information. Coordinates are in NAD83 datum

| Site ID | NWIS site ID | NWIS site name |
| --- | --- | --- |
| P1S1 | 380159121360601 | FRANKS TRACT SITE P1S1 NR BETHEL ISLAND CA |
| P1S2 | 380202121360801 | FRANKS TRACT SITE P1S2 NR BETHEL ISLAND CA |
| P1S3 | 380157121360501 | FRANKS TRACT SITE P1S3 NR BETHEL ISLAND CA |
| P2S1 | 380247121355301 | FRANKS TRACT SITE P2S1 NR BETHEL ISLAND CA |
| P2S2 | 380249121355701 | FRANKS TRACT SITE P2S2 NR BETHEL ISLAND CA |
| P3S1 | 380140121361501 | FRANKS TRACT SITE P3S1 NR BETHEL ISLAND CA |
| P3S2 | 380142121362301 | FRANKS TRACT SITE P3S2 NR BETHEL ISLAND CA |
| P3S3 | 380136121362001 | FRANKS TRACT SITE P3S3 NR BETHEL ISLAND CA |
| P4S1 | 380319121352201 | FRANKS TRACT SITE P4S1 NR BETHEL ISLAND CA |
| P4S2 | 380322121352101 | FRANKS TRACT SITE P4S2 NR BETHEL ISLAND CA |
| P5S1 | 380317121355801 | FRANKS TRACT SITE P5S1 NR BETHEL ISLAND CA |
| P6S1 | 380245121371201 | FRANKS TRACT SITE P6S1 NR BETHEL ISLAND CA |
| P6S2 | 380244121370701 | FRANKS TRACT SITE P6S2 NR BETHEL ISLAND CA |
| P6S3 | 380244121370702 | FRANKS TRACT SITE P6S3 NR BETHEL ISLAND CA |
| P7S1 | 380244121373501 | FRANKS TRACT SITE P7S1 NR BETHEL ISLAND CA |
| P7S2 | 380245121373901 | FRANKS TRACT SITE P7S2 NR BETHEL ISLAND CA |

References

Arar, E.J. and Collins, G.B., 1997, Method 445.0: In vitro determination of chlorophyll a and pheophytin a in marine and freshwater algae by fluorescence: Washington, D.C., U. S. Environmental Protection Agency, Office of Research and Development, National Exposure Research Laboratory.

https://cfpub.epa.gov/si/si_public_record_report.cfm?Lab=NERL&dirEntryId=309417.

Hansen, A.M., Kraus, T.E.C., Pellerin, B. A., Fleck, J. A., Downing, B. D., Bergamaschi, B. A., 2016, Optical properties of dissolved organic Matter (DOM): effects of biological and photolytic degradation: Limnology and Oceanography, v. 61, p. 1015-1032, https://doi.org/10.1002/lno.10270.

Hansen, A. M., Fleck, J. A., Kraus, T. E. C., Downing, B. D., Von Dessonneck, T., Bergamaschi, B. A., 2018, Procedures for using the Horiba Scientific Aqualog® fluorometer to measure absorbance and fluorescence from dissolved organic matter: U. S. Geological Survey Open-File Report 2018-1096, 31 p., https://doi.org/10.3133/ofr20181096.

Lunetta, R. S., Schaeffer, B. A., Stumpf, R. P., Keith, D., Jacobs, S. A., and Murphy, M. S. (2015). Evaluation of cyanobacteria cell count detection derived from MERIS imagery across the eastern USA. Remote Sensing of Environment 157, 24–34. https://www.sciencedirect.com/science/article/pii/S0034425714002211.

Matthews, M. W., Bernard, S., and Robertson, L. (2012). An algorithm for detecting trophic status (chlorophyll-a), cyanobacterial-dominance, surface scums and floating vegetation in inland and coastal waters. Remote Sensing of Environment 124, 637–652. https://www.sciencedirect.com/science/article/pii/S0034425712002350.

Mobley, C. D., Werdell, J., Franz, B., Ahmad, Z., and Bailey, S. (2016). Atmospheric Correction for Satellite Ocean Color Radiometry. Available at: https://ntrs.nasa.gov/citations/20160011399 [Accessed July 12, 2023].

Mobley, C. D. (1999). Estimation of the remote-sensing reflectance from above-surface measurements. Appl. Opt. 38, 7442. https://opg.optica.org/ao/viewmedia.cfm?uri=ao-38-36-7442&seq=0&html=true.

Potter, B. and Wimsatt, J., 2012, USEPA method 415.3: quantifying TOC, DOC, and SUVA: Journal American Water Works Association, v. 104, 6, p. E358-E369, https://doi.org/10.5942/jawwa.2012.104.0086.

Ruddick, K. G., Voss, K., Banks, A. C., Boss, E., Castagna, A., Frouin, R., et al. (2019a). A Review of Protocols for Fiducial Reference Measurements of Downwelling Irradiance for the Validation of Satellite Remote Sensing Data over Water. Remote Sensing 11, 1742. https://www.mdpi.com/2072-4292/11/15/1742.

Ruddick, K. G., Voss, K., Boss, E., Castagna, A., Frouin, R., Gilerson, A., et al. (2019b). A Review of Protocols for Fiducial Reference Measurements of Water-Leaving Radiance for Validation of Satellite Remote-Sensing Data over Water. Remote Sensing 11, 2198. https://www.mdpi.com/2072-4292/11/19/2198.

Stumpf, R. P., Davis, T. W., Wynne, T. T., Graham, J. L., Loftin, K. A., Johengen, T. H., et al. (2016). Challenges for mapping cyanotoxin patterns from remote sensing of cyanobacteria. Harmful Algae 54, 160–173. https://www.sciencedirect.com/science/article/pii/S1568988315301839.

U.S. Geological Survey (2023). National Water Information System. doi: http://doi.org/10.5066/F7P55KJN.

Wynne, T., Meredith, A., Stumpf, R., Briggs, T., and Litaker, W. (2020). Harmful Algal Bloom Forecasting Branch Ocean Color Satellite Imagery Processing Guidelines, 2020 Update. https://repository.library.noaa.gov/view/noaa/30906.

Wynne, T. T., Stumpf, R. P., Tomlinson, M. C., Warner, R. A., Tester, P. A., Dyble, J., et al. (2008). Relating spectral shape to cyanobacterial blooms in the Laurentian Great Lakes. International Journal of Remote Sensing 29, 3665–3672. https://www.tandfonline.com/doi/full/10.1080/01431160802007640.