Assessing spatial variability of nutrients, phytoplankton, and related water quality constituents in the California Sacramento-San Joaquin Delta at the landscape scale: 2024 High resolution mapping surveys (Provisional Release)

Brinkman, J.D., O'Donnell, K., Richardson, E.T., Soto Perez, J., Sturgeon, C.L., Uebner, M.Q., Nakatsuka, K., Bouma Gregson, K., Kraus, T.E.C., and Bergamaschi, B.A., 2024, Assessing spatial variability of nutrients, phytoplankton, and related water quality constituents in the California Sacramento-San Joaquin Delta at the landscape scale: 2024 High-resolution mapping surveys (Provisional Release): U.S. Geological Survey data release, https://www.sciencebase.gov/catalog/item/66427d79d34e1955f5a41cf0.

# Methods

Broad-area, high-resolution, boat-based water quality mapping surveys of the Sacramento-San Joaquin Delta (Delta) and Suisun Bay were conducted in May, July, and October of 2024. Each survey was comprised of four successive days of high-resolution mapping. The regions covered included the north-south axis of the Delta from the Sacramento River north of Freeport to the Harvey Banks export facility located in the southern Delta, including Old and Middle Rivers; the east-west axis of the Delta from the Sacramento River at Rio Vista to the San Joaquin River at Stockton, including Disappointment Slough and the North and South Forks of the Mokelumne River; and the northern and western Delta from the northern reaches of the Cache Slough Complex to the Grizzly, Honker, and Suisun Bays. On each day, the route of the mapping survey was determined by weather, tides and other logistical considerations. Data were collected at high speed to minimize the extent to which tidal currents obscure nutrient and phytoplankton distributions.

## Sampling and analysis

### Underway high resolution (HR) water sampling and onboard analyses

Surveys were conducted during daylight hours from a moving boat, the USGS research vessel (R/V) George Aiken, on May 6th through 9th, July 22nd through 25th, and October 7th through 10th, 2024, following the approach described by Bergamaschi and others (2024), Richardson and others (2023b), and Stumpner and others (2020). For analysis, sample water was continuously pumped onto the boat using two SHURflo Aqua King II pumps at a pressure of 60 psi and rate of 5 GPM (SHURflo Pentair, London, United Kingdom) while underway at speeds up to 30 miles per hour (mph) (13 meters per second [m/s]) using a pick-up tube mounted at a fixed depth of approximately 1 m below the surface.  Water was routed through a 178 micron (um) in-line strainer (Cole Parmer; EW-29595-47, Vernon Hills, Illinois) to remove large debris. Sample water was split into separate flow paths as described below, and each flow path was routed through a pressure-compensated manifold that maintained system pressure at a prescribed level (~20 psi) irrespective of boat speed (Bergamaschi and others, 2024). A 2-stage debubbler was used to remove bubbles that can interfere with instrument performance.  Flow rates were maintained by metering the discharge line, except for the flow path serving the open split interface, which was metered on the feed line. Flows were continuously monitored using a sight gauge (0.1-1 GPM Flow Meter LZM-15 with adjustable knob) for each flow path.

A manifold delivered unfiltered water to the following flow paths: (1) A flow-through system consisting of a thermosalinograph that recorded temperature, conductance, and salinity (Sea-Bird Scientific SB45 (TSG), Bellevue, Washington); fluorometers that measured fluorescence of chlorophyll (fCHL) and fluorescence of dissolved organic matter (fDOM, Sea-Bird Scientific WETstar (WS), Philomath, Oregon); a beam transmissometer that recorded transmittance (Sea-Bird Scientific model C-Star transmissometer (CStar), Philomath, Oregon); a nitrate (NO3) analyzer (SUNA V2; Sea-Bird Scientific, Bellevue, Washington). (2) A second path provides water to a multiparameter water quality sonde (YSI EXO2; Xylem Inc. (EXO), Rye Brook, New York) equipped with sensors to measure temperature, specific conductance (SpC), turbidity, pH, dissolved oxygen (DO), fDOM and fCHL. (3) A third path passed water to a fluorometer designed to measure different algal classes (FluoroProbe III (FP); BBE Moldaenke, Schwentinental, Germany; Eijkelkamp, 2014; Beutler and others, 2002).

In addition to water measurements, air temperature was also collected. A Campbell Scientific 108 Air Temperature sensor (Campbell Scientific, Logan, Utah) was mounted under the roof at the stern of the boat near the manifold system, 2-3 m above the surface of the water. The heat shield is not used, but it is shaded by the roof to protect from direct sunlight, and close to the cabin of the boat to protect from wind.

All instrumentation was cleaned, and calibrations were checked prior to each use following the manufacturer's recommendation or as described above. Data for most instruments were recorded at 1-second frequency on a single data logger (CR6, Campbell Scientific, Logan, Utah) together with a timestamp and boat position obtained from a high-resolution global positioning system receiver (16X-HVS, Garmin, Olathe, Kansas). The FluoroProbe logged data internally and to the host software every 1-2 seconds. The Timberline NH4 analyzer was connected to a stand-alone computer and collected data through its native software. The Picarro water stable isotope analyzer collected and logged data through its native software to an internal computer. All data were displayed in real time, so scientists on board could respond when they observed changes relevant to study objectives, or if issues with flow or instrument performance were noted. Data collected in situ are referred to as high resolution (HR) parameters.

### Discrete water sampling (DS) and laboratory analyses

Discrete water samples (DS) were collected either while underway or–in the case of more extensive sample requirements–while stopped on station. Discrete samples were collected while underway by passing water from the pickup tube through a 0.2-micron filter into a sample bottle. At designated sampling stations (approximately 10 per day) samples for determination of ammonium (NH4), nitrite (NO2), nitrate (NO3), phosphate (PO4), dissolved organic carbon (DOC), total dissolved nitrogen (TDN), and total dissolved phosphorus (TDP) concentrations were collected using this system. Additionally, unfiltered samples were collected approximately from 1 m depth using a submersible pump for analysis of chlorophyll a (Chl a), pheophytin, particulate carbon (PC), and particulate nitrogen (PN) concentration, and phytoplankton and picophytoplankton enumeration. Samples for phytoplankton enumeration and picophytoplankton enumeration were fixed immediately with Lugol’s iodine solution (~5%) and formaldehyde (~2%), respectively.

Most samples were stored on wet ice in the dark during transit to the laboratory where they were stored at 2 degrees Celsius (°C). Samples for picophytoplankton analysis were stored on dry ice in the dark during transit to the laboratory where picophytoplankton samples were stored at -80°C. Dissolved organic carbon samples were preserved to <2 pH with high-purity sulfuric acid the day following collection and stored at 2°C. Samples for pigment analysis were filtered within 24 hours of collection and immediately frozen (-80°C). Total Chl a samples were filtered through a 0.7 um nominal pore size glass fiber filter (Advantec MFS, Inc, Dublin, California). Chl a associated with the larger sized cell fraction were filtered through a 5 um pore size 25 mm polycarbonate filter (General Electric Healthcare, Chicago, Illinois). Samples for determination of PC and PN were filtered through 0.7 um nominal combusted glass fiber filter. Frozen Chl a filters and chilled nutrient samples were shipped to the Chesapeake Biological Laboratory (Solomons, Maryland). Phytoplankton and picophytoplankton enumeration samples were shipped to BSA Environmental Services, Inc. (Beachwood, Ohio) for analysis.

Concentrations of nutrients were determined colorimetrically using an automated discrete photometric analyzer. Concentration of nitrogen as nitrite (NO2-N) was determined by colorimetric analysis after diazotization with sulfanilamide (USEPA, 1993a).  For nitrate plus nitrite (NO3-N + NO2-N), samples were analyzed using enzymatic reduction (ASTM, 2014), and if results were below the reporting limit of 0.15 milligrams per liter (mg/L), they were rerun using cadmium reduction (USEPA, 1993a) due to the lower detection limit of that method. Concentration of ammonium as nitrogen (NH4-N) was determined by colorimetric analysis after reaction with alkaline phenol-hypochlorite (Rice and others, 2017). Orthophosphate as phosphorus (PO4-P; also referred to as soluble reactive phosphorus) was determined by colorimetric analysis after reaction with ammonium molybdate and reduction with ascorbic acid (USEPA, 1993b). Total dissolved nitrogen (TDN) and total dissolved phosphorus (TDP) were determined by alkaline persulfate digestion (USEPA 1993a, 1993b).

For particulate carbon (PC) and particulate nitrogen (PN) analyses, filters were first dried at approximately 47°C to a constant weight. Then, a weighed amount of dried particulate was combusted at high temperature and measured using an elemental analyzer (Zimmerman and others, 1997). Chlorophyll a and phaeophytin concentrations were determined fluorometrically according to EPA Method 445.0 (Arar and Collins, 1997), using mechanical grinding to macerate cells.

Dissolved organic carbon concentrations were measured using a total organic carbon analyzer (TOC-VCSH, Shimadzu Scientific Instruments, Columbia, Maryland) using high-temperature catalytic combustion according to a modified version of USEPA method 415.3 (Potter and Wimsatt, 2012). Absorbance spectra and fluorescence EEMs were simultaneously measured using an Aqualog® equipped with a photodiode and charge-coupled device (CCD) (Horiba Instruments, NJ, USA) according to the method described by Hansen and others (2018). Briefly, excitation and absorbance spectra were measured using a 150 W xenon lamp, a 5 nm bandpass, and a 1 s integration time at wavelengths of 240–600 nm. Emission spectra were collected with a CCD at approximately 1.64 nm (4 pixel) intervals at wavelengths of 250–600 nm. To reduce ultraviolet light exposure of the sample and limit the effects of photobleaching, excitation and absorbance wavelengths were scanned from low to high energy (i.e., visible to ultraviolet range) during analysis. Peaks, ratios, and slopes reported in the data were calculated as described by Hansen and others (2016).

Phytoplankton enumeration was completed by BSA Environmental Services in accordance with the American Public Health Association (APHA) Standard Method 10200 (2023). This method focuses on counting and identifying microplankon (20-200 microns) and nanoplankton (2 to 20 microns). Briefly, phytoplankton were enumerated to the lowest possible taxonomic level using membrane-filtered slides, as described in McNabb (1960), and counted using a Leica DMLB compound microscope. The magnification used (100X - 1000X) depended upon the size of the dominant taxa and presence of particulates to ensure that enumeration and identification of taxa of various sizes was achieved. A minimum of 400 natural units (colonies, filaments, and unicells) or a minimum of 50 fields were counted from each sample. Counting 400 natural units provides accuracy within 90-percent confidence limits (Lund and others, 1958). In addition, an entire strip of the filter was counted at high magnification (usually 630X) along with one-half of the filter at a lower magnification (usually 400X) to ensure complete species reporting. Cell biovolume of all identified taxa were estimated using formulae for solid geometric shapes that most closely match the cell shape (Hillebrand and others, 1999). Biovolume calculations were based on measurements of 10 organisms per taxon for each sample, where possible. For taxa with substantial size variation (such as diatoms), size classes were designated arbitrarily to determine average cell size (biovolume). The mean biovolume within each size class were used to calculate the total biovolume contributed by the taxon to its representative sample (Burkholder and Wetzel, 1989). Additional phytoplankton enumeration data collected in the Delta can be found at https://doi.org/10.5066/P97ZBPLH (Richardson and others, 2023a).

Picophytoplankton were enumerated via epifluorescence microscopy as described by Murrell and Lores (2004) using a Leica DM2500 epifluorescence microscope equipped with a Xenon light source as well as multiple excitation and emission filter sets at 1000X total magnification. Phycoerythrin-containing cells (PE) and phycocyanin-containing cells (PC) were counted and measured separately and then summed to obtain total picophytoplankton densities and biovolume for cells sized between 0.2 and 2.0 microns. For each sample, a minimum of 300 cells were counted unless this tally was not reached within 100 fields.

Discrete sample results will be added to this data release as they become available.

## Data processing

All data were processed using the pandas software library (0.220; McKinney, 2010) in Python (3.9.13; Van Rossum and Drake, 2009) to remove periods of compromised data (e.g. flow blockages, bubbles), to apply instrument corrections and unit conversions, and to apply a centered 20 second median filter to the time series.

### Spatial Alignment (SA)

High-resolution (HR) data were spatially aligned (SA) to the North American Datum 1983 [NAD 1983 (2011)] geographic coordinate system to facilitate comparisons between dates and to minimize the effects of differential data density on the spatial interpolation calculations. The spatial alignment was performed using the GeoPandas software library (0.6.1; Jordahl, 2020) in python, which assigned median-filtered data points to polygon shapefiles that are spaced at approximately 150 m intervals. Spatially aligned data were then aggregated by polygon shapefile to obtain summary statistics of all median-filtered data within a polygon.

Interpolated water quality data were created from the spatially aligned data using ESRI’s ArcGIS Pro (2.2.1) Spline with Barriers tool (Terzopoulos and Witkin, 1988). The raster output was smoothed with the Focal Statistics tool, and interpolated values from the raster output were extracted to the spatially aligned points with the Extract Multi Values to Points tool to create continuous spatially aligned data across the mapping domain. Raster values were converted to vector format using the Contour tool, then clipped with the Clip tool using a Sacramento-San Joaquin Delta polygon obtained from the National Hydrography Dataset (U.S. Geological Survey, 2018). These data are referred to as spatially aligned (SA) and are associated with a specific sampling day, and water quality parameter.

# References

American Public Health Association (APHA), American Water Works Association, Water Environment Federation, Lipps W.C., Braun-Howland E.B., Baxter T.E., eds., 2023, Standard Methods for the Examination of Water and Wastewater, 24th ed, Washington DC: APHA Press.

Arar, E.J. and Collins, G.B., 1997, Method 445.0: In vitro determination of chlorophyll a and pheophytin a in marine and freshwater algae by fluorescence: Washington, D.C., U. S. Environmental Protection Agency, Office of Research and Development, National Exposure Research Laboratory. [Also available at https://cfpub.epa.gov/si/si_public_record_report.cfm?Lab=NERL&dirEntryId=309417.]

ASTM Standard D7781-14, 2014, “Standard Test Method for Nitrite-Nitrate in Water by Nitrate Reductase,” ASTM International, West Conshohocken, PA.

Bergamaschi, B.A., Kraus, T.E.C., Downing, B.D., Stumpner, E.B., O’Donnell, K., Hansen, J.A., Soto Perez, J., Richardson, E.T., Hansen, A.M., and Gelber, A., 2024, Assessing spatial variability of nutrients, phytoplankton, and related water-quality constituents in the California Sacramento–San Joaquin Delta at the landscape scale—2018 high resolution mapping surveys: U.S. Geological Survey Scientific Investigations Report 2023–5060, 47 p., https://doi.org/10.3133/sir20235060.

Beutler, M., Wiltshire, K., Meyer, B., Moldaenke, C., Lüring, C., Meyerhöfer, M., & Dau, H., 2002, A fluorometric method for the differentiation of algal populations in vivo and in situ: Photosynthesis Research, v. 72, p. 39-53,  https://doi.org/10.1023/A:1016026607048.

Burkholder, J.B. and Wetzel, R.G., 1989, Microbial colonization on natural and artificial macrophytes in a phosphorus-limited, Hardwater Lake: Journal of Phycology, v. 25, p. 55-65, https://doi.org/10.1111/j.0022-3646.1989.00055.x.

Eijkelkamp, 2014. FluoroProbe User Manual. V. E2, accessed March 15, 2017.

Hansen, A.M., Kraus, T.E.C., Pellerin, B. A., Fleck, J. A., Downing, B. D., Bergamaschi, B. A., 2016, Optical properties of dissolved organic Matter (DOM): effects of biological and photolytic degradation: Limnology and Oceanography, v. 61, p. 1015-1032, https://doi.org/10.1002/lno.10270.

Hansen, A. M., Fleck, J. A., Kraus, T. E. C., Downing, B. D., Von Dessonneck, T., Bergamaschi, B. A., 2018, Procedures for using the Horiba Scientific Aqualog® fluorometer to measure absorbance and fluorescence from dissolved organic matter: U. S. Geological Survey Open-File Report 2018-1096, 31 p., https://doi.org/10.3133/ofr20181096.

Hillebrand, H., Dürselen, C., Kirschtel, D., Pollingher, U., and Zohary, T., 1999, Biovolume calculation for pelagic and benthic microalgae: Journal of Phycology v. 35, p. 403-424, https://doi.org/10.1046/j.1529-8817.1999.3520403.x.

Jordahl, K., 2020, GeoPandas: Python tools for geographic data, accessed December 2, 2020, at https://doi.org/10.5281/zenodo.3946761.

Lund, J. W. G., Kipling, C., Le Cren, E. D., 1958, The inverted microscope method of estimating algal numbers and the statistical basis of estimations by counting: Hydrobiologia, v. 11, p. 143-170. https://doi.org/10.1007/BF00007865.

McKinney, W., 2010, Data structures for statistical computing in python: Proceedings of the 9th Python in Science Conference, v. 445, p. 51–56, https://doi.org/10.25080/Majora-92bf1922-00a.

McNabb, C. D., 1960, Enumeration of freshwater phytoplankton concentrated on the membrane filter: Limnology and Oceanography, v. 5, p. 57-61, https://doi.org/10.4319/lo.1960.5.1.0057.

Murrell, M.C. and Lores, E.M., 2004, Phytoplankton and zooplankton seasonal dynamics in a subtropical estuary: importance of cyanobacteria: Journal of Plankton Research, v. 26, p. 371-382, https://doi.org/10.1093/plankt/fbh038.

Potter, B. and Wimsatt, J., 2012, USEPA method 415.3: quantifying TOC, DOC, and SUVA: Journal American Water Works Association, v. 104, 6, p. E358-E369, https://doi.org/10.5942/jawwa.2012.104.0086.

Rice, E., Baird, R., and Eaton, A., 2017, 4500-NH3 nitrogen (ammonia) G: Standard methods for examination of water and wastewater. American Public Health Association, American Water Works Association, and Water Environment Federation, Washington, DC.

Richardson, E.T., Bouma-Gregson, K., Kraus, T.E.C., O'Donnell, K., Sturgeon, C.L., Soto Perez, J., Delascagigas, A., Nakatsuka, K.K., Burau, D.J., Gelber, A.D., Von Hoyningen Huene, B.L., Jumps, N.I., and Bergamaschi, B.A., 2023a, Phytoplankton Species Composition and Abundance in the Sacramento-San Joaquin River Delta: Microscopic Enumeration of USGS Samples, beginning in 2016 (ver 1.1, December 2023): U.S. Geological Survey data release, https://doi.org/10.5066/P97ZBPLH.

Richardson, E.T., Hansen, A.M., Kraus, T.E., Downing, B.D., Forsberg, D., Stillian, J., O’Donnell, K., Sturgeon, C.L., and Bergamaschi, B.A., 2023b, A novel boat‐based field application of a high‐frequency conductometric ammonium analyzer to characterize spatial variation in aquatic ecosystems: Limnology and Oceanography: Methods, https://doi.org/10.1002/lom3.10579.

Stumpner, E. B., Bergamaschi, B. A., Kraus, T. E. C., Parker, A. E., Wilkerson, F. P., Downing, B. D., Dugdale, R. C. Murrell, M. C., Carpenter, K. D., Orlando, J. L., and Kendall, C., 2020, Spatial variability of phytoplankton in a shallow tidal freshwater system reveals complex controls on abundance and community structure: Science of The Total Environment, v. 700, no. 134392, 17 p., https://doi.org/10.1016/j.scitotenv.2019.134392.

Terzopoulos, D. and Witkin, A., 1988, Physically based models with rigid and deformable components: IEEE Computer Graphics and Applications, v. 8, no. 6, p. 41-51, https://doi.org/10.1109/38.20317.

USEPA, 1993a, Method 353.2: Determination of nitrate‐nitrite nitrogen by automated colorimetry: Environ. Monitoring Systems Lab.

USEPA, 1993b, Method 365.1: Determination of Phosphorus by Semi-Automated Colorimetry: Methods for chemical analysis of water and wastes.

U.S. Geological Survey, 2018, National Hydrography Dataset (NHD): U. S. Geological Survey, National Geospatial Technical Operations Center (NGTOC), accessed December 14, 2018, at https://www.usgs.gov/national-hydrography/national-hydrography-dataset.

U.S. Geological Survey, 2022, USGS Water Data for the Nation: U.S. Geological Survey, National

Water Information System database, accessed September 16, 2024 at https://doi.org/10.5066/F7P55KJN.

Van Rossum, G., and Drake, F. L., 2009, Python 3 Reference Manual. Scotts Valley, CA: CreateSpace, accessed March 15, 2017.

Zimmermann, C.F., Keefe, C.W., and Bashe, J., 1997, Method 440.0: Determination of Carbon and Nitrogen in Sediments and Particulates of Estuarine/Coastal Waters Using Elemental Analysis: United States Environmental Protection Agency, Office of Research and Development, National Exposure Research Laboratory.